Objective3:

Task 3.1: We aim to develop a model predictive control (MPC) framework for BSS that exploits high-fidelity physics models to trade-off economic incentives from energy market arbitrage and long-term degradation effects. Instead of simply correlating battery degradation with the number of charging cycles, the proposed framework will leverage a first-principles model that accurately predicts the degradation level and the remaining capacity of battery packs. This model considers rapid fluctuations in current density, ion transfer, and heat transfer within the battery, which are induced by frequent charging and discharging cycles. MPC provides a robust decision-making logic where conditions, such as battery safety, state of charge, EV customer service level are explicitly incorporated as constraints in the MPC optimization framework. The framework determines the optimal energy charging and discharging decisions at each time interval, following a rolling horizon approach, with the objective of maximizing the total annual profit of the BSS. This model is of industrial relevance as it enables BSS to participate in fast-changing electricity markets while maintaining asset integrity.

Task 3.2: We aim to extend the deterministic MPC of task 3.1 (which assumes perfect information on energy prices and EV customer demand) to a stochastic MPC that explicitly accounts for uncertainties in future electricity prices and battery demand. We will first develop short-term forecasting models (e.g., with a 24-hour time horizon) for electricity prices and EV battery demand, using autoregressive (AR) models or Gaussian processes (GP) to predict means and quantify uncertainties. Given the probabilistic distributions of future electricity prices and EV battery demand, we will construct a stochastic MPC framework that incorporates these uncertainties into decision-making. This framework enables real-world implementation, where electricity prices and battery demand are not known with certainty at the time of decision-making. Stochastic MPC provides a more systematic approach to handling a wide range of disturbances caused by market fluctuations and demand variability. We will benchmark the performance of stochastic MPC against deterministic MPC to highlight the importance of accounting for uncertainties in optimizing the BSS operations.